# PURPOSE

This advisory explains how to—

## Determine whether the repair station operations or quality manuals and other documents show compliance with the Revision 2 to the Special Conditions contained in the U.S.-Canadian Maintenance Implementation Procedures (MIP) dated Sept. 9, 2025.

## Request authority to work on articles destined to be installed on Canadian-registered products under a FAA foreign repair station certificate.

# APPLICABILITY

14 CFR part 145 air agencies (repair stations) working on products or articles under the jurisdiction of the Canadian TCCA.

# DEFINITIONS AND ACRONYMS

## AC = Advisory Circular

## Article = material, part, component, process, or appliance (IPA, Article 1, paragraph 1.12.10.)

## BASA = Bilateral Aviation Safety Agreement (BASA)

## CARs = Canadian Aviation Regulations

## CAWIS = Continued Airworthiness Web Information System

## CFR = Code of Federal Regulations

## FAA = Federal Aviation Administration

## FM = Form(s) Manual

## IPA = Implementation Procedures for Airworthiness

## MIP = Maintenance Implementation Procedures

## Product = aircraft, aircraft engine, or propeller (14 CFR § 21.1(b)(6))

## RSQM = Repair Station Quality Manual

## TCCA = Transport Canada Civil Aviation

## IPA = Implementation Procedures for Airworthiness (Revision 3, dated April 26, 2021)

## TPM = Training Program Manual

# REFERENCES AND RELATED DOCUMENTS

## U.S.-Canadian BASA, Article 1

## MIP, Section B—Special Conditions: Guidance for FAA-certificated A&P Mechanics and FAA-certificated Repair Stations

## U.S.-Canadian IPA, Section II, paragraph 2.2

## AC 43-10C United States—Canadian Bilateral Aviation Safety Agreement Maintenance Implementation Procedures

## CAR, Part V – Airworthiness, Standard 571

## CAR, Part VII – Commercial Air Services, Standard 700

## AC No. 571-002 – Subject: Maintenance Implementation Procedures – Bilateral Aviation Safety Agreement Between Canada and the United States

# BACKGROUND

The FAA and Transport Canada amended the procedures applicable to FAA-certificated repair stations and Canadian approved maintenance organizations under the Article 1 of the countries’ MIP; revision 2. The revision became effective January 7, 2026.

# GENERAL

Although the MIP does not require all repair stations to develop a supplement, the associations recognize that there are elements that impact all repair stations.

The matrices in this Advisory are to be used to show compliance with the requirements of the MIP before performing work on Canadian-registered aircraft or the aircraft engines, propellers, or articles that will be installed on those aircraft.

- The matrix in Paragraph (X) is for FAA-certificated persons performing work on Canadian-registered aircraft the aircraft engines, propellers, or articles destined for installation on those aircraft that are not in commercial service.

- The matrix in Paragraph (XI) is for FAA-certificated persons performing work on Canadian-registered aircraft in commercial service under Part VII of the CARs.

- The matrix in Paragraph (XII) is for FAA-foreign certificate persons that wish to perform work on aircraft engines, propellers, or articles destined for installation on Canadian-registered aircraft.

Use only the one that is applicable to the repair station ratings and customer base.

Each time the MIP changes it should be reviewed to ensure continued compliance.

# INSTRUCTIONS FOR INITIAL COMPLETION

## For each special condition listed in the appropriate matrix, input the RSQM, TPM, FM, form or related procedure or documentation that establishes compliance with the element in the Compliance Reference column.

## When an element does not apply, explain why so when questions are asked, the answer and reasoning are readily apparent.

# INSTRUCTIONS FOR UPDATING

## When the MIP changes, save the matrix with the new revision level and the date the new MIP will become effective (which will be noted in the MIP).

## Turn on tracked changes or accept all previous changes but keep tracking changes or otherwise ensure revision control.

## Change the footers in the matrix to the revision level and the date the MIP will become effective (which will be noted in the MIP).

## Review each paragraph and track any changes, including any references to documents showing compliance that may need to be added. When an element does not apply, explain why so when questions are asked, the answer and reasoning are readily apparent.

# CONTACT INFORMATION

| Aircraft Electronics Association |  | Aeronautical Repair Station Association |
| --- | --- | --- |
| 3570 NE Ralph Powell Road Lee’s Summit, MO 64064-2360 info@aea.net |  | 121 North Henry Street Alexandria, VA 22314-2903 arsa@arsa.org |

# Special Condition—Section B—Paragraph 2

This Special Condition applies to FAA-certificated Repair Stations Located Within The United States Performing Work On Canadian-registered Aircraft That Are Not Operated In Commercial Air Service Pursuant To Part VII Of The CARs and/or Aeronautical Products.

Although no supplement is required, knowledge and compliance is established by completion of this matrix.

|  |  | Canadian MIP |  | Compliance Reference |
| --- | --- | --- | --- | --- |
| 2.2 |  | An FAA-certificated repair station may perform maintenance and modifications on Canadian-registered aircraft that are not operated in commercial air service pursuant to Part VII of the CARs, and aeronautical products of those aircraft, under TCCA regulatory control for the purpose of this MIP. The FAA-certificated repair station may also approve that aircraft for return to service if it complies with the following special conditions: |  |  |
| a. | The FAA-certificated repair station and Canadian-registered aircraft and/or aeronautical product must be located within the United States. |  |
| b. | The FAA-certificated repair station must perform the work within its ratings and limitations contained in the 14 CFR part 145 certificate and operations specifications or current capability list. The FAA-certificated repair station is not authorized to perform annual inspections on Canadian-registered aircraft. |  |
| c. | Technical data used to perform major repairs or major modifications must be data approved by TCCA as described in CAR 571.06. FAA field approvals supporting modifications using FAA Form 337 are not acceptable on Canadian-registered aircraft. Please refer to the FAA/TCCA Implementation Procedures for Airworthiness for further information. |  |
| d. | The FAA-certificated repair station may perform “Specialized Maintenance” in accordance with Schedule II of CAR 571.04, except on Canadian-registered aircraft that are operated in commercial air service pursuant to Part VII of the CARs, as a TCCA Supplement is required. | The repair station does not work on aircraft in commercial air services. |
| e. | Upon completion of a major repair or modification on TCCA aircraft or article, the FAA-certificated repair station shall submit a major repair or modification report to TCCA as required by CAR 571.12. The report must be sent to the Transport Canada Center assigned to the geographical area in which the owner of the aircraft resides or manages his business within 30 days after the aircraft is returned to service. The submitter should consult with the owner of the aircraft or article to determine which Transport Canada Center the report should be sent to. A copy of the report shall be included in the aircraft records. |  |
| f. | Maintenance and modifications on aircraft, engines, propellers, appliances, or components must be accomplished using the methods, techniques, and practices prescribed in the current manufacturer’s applicable manuals. Where applicable, this work may also be accomplished in accordance with current ICA issued by the manufacturer or other work instructions acceptable to the TCCA. |  |
| g. | All parts and components installed during the performance of work, other than standard or commercial parts, must be accompanied by the appropriate authorized release certificate. This could be an FAA Authorized Release Certificate – Form 8130-3 (from an FAA-certificated repair station located in the United States or an FAA-certificated repair station located outside of the United States that is accepted in accordance with this MIP) or a TCCA Authorized Release Certificate – Form One, Statement of Conformity, or equivalent as provided for pursuant to an agreement with TCCA. A rebuilt engine may be installed on a Canadian-registered aircraft. Rebuilt parts are not acceptable for installation. |  |
| h. | For the purposes of approval for return to service, the recording of maintenance and modifications must meet the requirements of 14 CFR part 43 (§§ 43.9 and 43.11). |  |
| i. | The FAA-certificated repair station may be subject to an inspection/investigation by the FAA, upon the request of TCCA, for continued compliance with relevant regulatory requirements. |  |
| j. | Serious defects, malfunctions, or unairworthy conditions on aeronautical products under TCCA regulatory control must be reported to TCCA within 3 days. FAA-certificated repair stations should report Suspected Unapproved Parts (SUP) to TCCA by using the SDR via Form 24-0038, which is available online at the following address: http://wwwapps.tc.gc.ca/Corp-Serv-Gen/5/forms-formulaires/download/24- 0038_BO_PD or by registering with the CAWIS website and submitting the report on-line. The CAWIS site is found here: https://wwwapps.tc.gc.ca/saf-sec-sur/2/cawis-swimn/i.aspx?lang=eng. |  |
| 2.3 | The FAA-certificated repair station working on non-commercial Canadian-registered aircraft or aeronautical product under TCCA regulatory control, for the purpose of this MIP, does not require additional approval from the FAA or TCCA. |  |  |  |

# Special Condition—Section B—Paragraph 3

This Special Condition applies to FAA-certificated Repair Stations Located Within the United States Performing Work On Canadian-registered Aircraft That Are Operated In Commercial Air Service Pursuant To Part VII Of The CARs. A TCCA Supplement Is Required (Including For Line Maintenance Facilities Located Outside The United States).

The Matrix provides assurance that the Supplement contains the required elements.

|  |  | Canadian MIP |  |  | Compliance Reference |
| --- | --- | --- | --- | --- | --- |
| 3.2 |  | An FAA-certificated repair station performing maintenance and modifications on aircraft that are operated in commercial air service pursuant to Part VII of the CARs will have in its FAA-accepted Repair Station Manual (RSM) a supplement describing the procedures that will ensure compliance with following special conditions: |  |  | The repair station works on Canadian-registered aircraft operated in commercial air services. |
| a. | Confirmation that the FAA-certificated repair station and Canadian-registered aircraft are located within the United States. Except that line maintenance may be performed outside the United States as described in Paragraph 3.2(b) below. |  |  |
| b. | Where a 14 CFR part 145 repair station intends on performing line maintenance outside of the United States and Canada, procedures to ensure that: |  |  |
| 1. | An operations specification D107 authorizes the repair station to perform line maintenance and lists the specific locations line maintenance is performed. |  |
| 2. | It is performed in accordance with the RSM and identified in the accepted supplement. |  |
| 3. | It is not performed in countries where existing agreements or arrangements are in place between that country and Canada for the performance or acceptance of maintenance. Refer to https://tc.canada.ca/en/aviation/aircraft-airworthiness/international-agreements-arrangements for current Canadian agreements and arrangements. |  |
| c. | Procedures to ensure compliance with the air operator's work order or contract, and other mandatory instructions contained in the air operator’s MCM. |  |  |
| d. | Procedures to ensure data approved by TCCA is used to perform major repairs or major modifications as described in CAR 571.06. FAA field approvals are not acceptable on Canadian-registered aircraft and aeronautical products under TCCA regulatory control, including those documented on an FAA Form 8110-3. |  |  |
| e. | All parts and components installed during the performance of work, other than standard or commercial parts, must be accompanied by the appropriate authorized release certificate. This could be an FAA Authorized Release Certificate – Form 8130-3 (from an FAA-certificated repair station located in the United States or an FAA-certificated repair station located outside of the United States that is accepted in accordance with this MIP) or a TCCA Authorized Release Certificate – Form One, Statement of Conformity, or equivalent as provided for pursuant to an agreement with TCCA. A rebuilt engine may be installed on a Canadian-registered aircraft. Rebuilt parts are not acceptable for installation. |  |  |
| f. | Procedures to ensure that upon completion of a major repair or modification on TCCA aircraft or article, the FAA-certificated repair station shall submit a major repair or modification report to TCCA as required by CAR 571.12. The report must be sent to the Transport Canada Center assigned to the geographical area in which the owner of the aircraft resides or manages their business within 30 days after the aircraft is returned to service. The submitter should consult with the owner of the aircraft or article to determine which Transport Canada Center the report should be sent to. A copy of the report shall be included in the aircraft records. |  |  |
| g. | Procedures to ensure compliance with the ICA in accordance with the air operator’s MCM and approved maintenance schedule. |  |  |
| h. | Procedures to ensure that work that disturbs engine or flight controls will be subject to an independent check, as required by CAR 571. The system that has been disturbed needs to be inspected by at least two persons for correct assembly and correct locking of any parts disturbed by the maintenance performed, including an operational check for proper sense and range of motion of the engine or flight controls. The technical record will contain the signatures of both persons who performed the check. One of the signatures required by this section may be that of the person who has signed the maintenance release. |  |  |
| i. | Procedures to ensure that all employees working on aircraft operated per CAR Part VII have been trained on the TCCA Supplement. |  |  |
| j. | Procedures to ensure that each employee assigned to perform a maintenance task is trained and authorized to perform the assigned task. |  |  |
| k. | Procedures to ensure serious defects, malfunctions, or unairworthy conditions on aeronautical products under TCCA regulatory control must be reported to TCCA within 3 days. FAA-certificated repair stations should report Suspected Unapproved Parts (SUP) to TCCA by using the SDR via Form 24-0038, which is available online at the following address: http://wwwapps.tc.gc.ca/Corp-Serv-Gen/5/forms-formulaires/download/24-0038_BO_PD or by registering with the CAWIS website and submitting the report on-line. The CAWIS site is found here: https://wwwapps.tc.gc.ca/saf-sec-sur/2/cawis-swimn/i.aspx?lang=eng. |  |  |
| l. | Procedures to ensure that each person who approves an aircraft for return to service or signs a maintenance release for an aircraft has received the required FAA training on the aircraft type. |  |  |
| m. | Procedures to ensure the incorporation of portions of the air operator’s Safety Management System (SMS). Specifically: |  |  |
| 1. | Establish the contact person within the FAA-certificated repair station with whom the Canadian air operator will communicate SMS-related information; |  |
| 2. | Establish and maintain a reporting system in accordance with the air operator/repair station-established contract maintenance agreement; |  |
| 3. | The reporting system should ensure the timely collection of information related to hazards, incidents and accidents, the investigation, analysis and identification of the cause or probable cause, risk management analyses, and relevant to issues that may adversely affect safety; and |  |
| 4. | The reporting system should identify, or adopt, a non-punitive reporting and emergency response plan, in accordance with the air operator/repair station-established contract maintenance agreement. |  |
| n. |  | Procedures to ensure the organization’s supplement remains current by submitting to the appropriate FAA FSO for acceptance within 60 days with respect to the following change scenarios: |  |  |
| 1. | MIP changes, |  |
| 2. | Organizational changes, or |  |
| 3. | Regulatory requirement changes that affect the organization. |  |
| These procedures should identify who within the organization is responsible for approving amendments and for ensuring that all amendments to the supplement are submitted to the FAA for acceptance. Failure to ensure that the supplement is kept up to date and that the repair station staff comply with the procedures therein could invalidate maintenance accomplished. |  |  |
| 3.3 |  | The supplement must be submitted to the FAA-certificated repair station’s responsible FAA office for acceptance. |  |  |  |  |

# Special Condition—Section B—Paragraph 4

This Special Condition applies to FAA-Certificated Repair Stations Located Outside the United States.

Although no supplement is required, the matrix provides assurance that the criteria of Section B, Paragraph 4 have been met so an appropriate authorization from TCCA can be obtained.

|  | Canadian MIP |  | Compliance Reference |
| --- | --- | --- | --- |
| 4.1 | TCCA special conditions are applicable to FAA-certificated repair stations located outside the United States. When requested, an FAA-certificated repair station located outside of the United States may perform maintenance and modifications on aeronautical products (this includes only components or sub-components, but not complete aircraft) under TCCA regulatory control based on an authorization given in accordance with the following criteria: |  | This repair station is not requesting authority to work on aircraft under TCCA jurisdiction. See attached air agency certificate, with all ratings and limitations including operations specifications and current capability list. |
| a. | The FAA-certificated repair station is located in a country that has not entered into a maintenance agreement with TCCA. Refer to https://tc.canada.ca/en/aviation/aircraft-airworthiness/international-agreements- arrangements for current TCCA maintenance agreements. | This repair station is/is not located in a country with which Canada (TCCA) has a bilateral—NAME OF COUNTRY |
| b. | The FAA-certificated repair station does not hold a TCCA foreign AMO approval in accordance with CAR 573.13. | True |
| 4.2 | In addition to the requirements specified above, an FAA-certificated repair station must follow the following special conditions when performing maintenance on a component or sub-component destined for installation on a Canadian-registered aircraft: |  | Compliance with each special condition is contained in the referenced sections of the FAA repair station, quality, forms, and training program: |
| a. | The FAA-certificated repair station must perform the work within its ratings and limitations contained in the 14 CFR part 145 certificate and operations specifications or current capability list. |  |
| b. | Technical data used to perform major repairs or major modifications must be data approved by TCCA as described in CAR 571.06. FAA field approvals supporting modifications using FAA Form 337 are not acceptable on aeronautical products under TCCA regulatory control. Please refer to the FAA/TCCA Implementation Procedures for Airworthiness for further information. |  |
| c. | The FAA-certificated repair station may perform “Specialized Maintenance” in accordance with Schedule II of CAR 571.04. |  |
| d. | Maintenance and modifications on aeronautical products (excluding aircraft) must be accomplished using the methods, techniques, and practices prescribed in the manufacturer’s applicable manuals. Where applicable, this work may also be accomplished in accordance with current ICA issued by the manufacturer or other work instructions acceptable to TCCA. |  |
| e. | All parts and components installed during the performance of work, other than standard or commercial parts, must be accompanied by the appropriate authorized release certificate. This could be an FAA Authorized Release Certificate – Form 8130-3 (from an FAA-certificated repair station located in the United States or an FAA-certificated repair station located outside of the United States that is accepted in accordance with this MIP) or a TCCA Authorized Release Certificate - Form One, Statement of Conformity, or equivalent as provided for pursuant to an agreement with TCCA. Rebuilt parts are not acceptable for installation. |  |
| f. | For the purposes of approval for return to service, the recording of maintenance and modifications must meet the requirements of 14 CFR part 43 (§§ 43.9 and 43.11). |  |
| g. | The FAA-certificated repair station may be subject to an inspection/investigation by the FAA, upon the request of TCCA, for continued compliance with relevant regulatory requirements. |  |
| h. | Serious defects, malfunctions, or unairworthy conditions on aeronautical products under TCCA regulatory control must be reported to TCCA within 3 days. FAA-certificated repair stations should report Suspected Unapproved Parts (SUP) to TCCA by using the SDR via Form 24-0038, which is available online at the following address: http://wwwapps.tc.gc.ca/Corp-Serv-Gen/5/forms-formulaires/download/24-0038_BO_PD or by registering with the CAWIS website and submitting the report on-line. The CAWIS site is found here: https://wwwapps.tc.gc.ca/saf-sec-sur/2/cawis-swimn/i.aspx?lang=eng |  |
| 4.3 | An FAA-certificated repair station working on components or sub-components under the regulatory control of TCCA for the purposes of this MIP does not require additional approval from the FAA. However, it does require an authorization from TCCA to ensure the criteria of this section has been met. |  | Advisory Circular (AC) No. 571-002 set forth the requirements for obtaining authorization. |

## Obtaining Authorization from TCCA

An FAA certificated repair station located outside the US must submit a request for authorization to maintain aeronautical aircraft engines, propellers, and articles under regulatory control of TCCA to the following address: faatccaopair-faatcacnavop@tc.gc.ca.

## Contents of the Request for Authorization

The letter requesting the authorization to perform maintenance on aircraft engines, propellers, and articles under regulatory control of TCCA must contain:

- A copy of the Special Condition—Section B—Paragraph 4 establishing that:

  - The FAA certificated repair station is not located in a country that has entered into a maintenance agreement with TCCA.

  - The FAA certificated repair station does not hold a TCCA foreign AMO approval in accordance with CAR 573.13.

  - The FAA certificated repair station complies will comply with all other special conditions when working on components or sub-components under TCCA control as is required by Special Condition—Section B—Paragraph 4, paragraph 4.2.

- A copy of the FAA issued foreign air agency certificate with all ratings and limitations including operations specifications and current capability list.

- Either:

  - A letter from the manufacturer stating that it recommends the FAA certificated repair station to maintain its product(s); or

  - Proof that the maintenance that is to be provided by the FAA-certificated foreign repair station is not available in Canada. The proof that the maintenance cannot be obtained in Canada can come from the customer, or by showing the FAA foreign repair station performs a specialized maintenance (see the tasks set out in Schedule II of Part V, Subpart 71 of the CARs) that is required to be performed on Canadian-registered products or articles.